| Название устройства | Интерфейс | Подсеть | IP | Маска | Шлюз |
| --- | --- | --- | --- | --- | --- |
| M | Fa0/0 | LAN_1 | 198.133.219.1 | 255.255.255.240 | - |
| ПК1 | Eth0 | LAN_1 | 198.133.219.14 | 255.255.255.240 | 198.133.219.1 |
| М | Fa0/1 | LAN_2 | 198.133.219.17 | 255.255.255.240 | - |
| ПК2 | Eth0 | LAN_2 | 198.133.219.29 | 255.255.255.240 | 198.133.219.17 |
| ПКЗ | Eth0 | LAN_2 | 198.133.219.30 | 255.255.255.240 | 198.133.219.17 |

198.133.219.0/24

255.255.255.0

.0000 0000

Подсети Узлы

| # | Маска | Адрес подсети | Диапазон | Широковещатель |
| --- | --- | --- | --- | --- |
| 1 | 0000 | 198.133.219.0/28 | 198.133.219.[1 -14] | 198.133.219.15 |
| 2 | 0001 | 198.133.219.16/28 | 198.133.219.[17 - 30] | 198.133.219.31 |
| 3 | 0010 | 198.133.219.32/28 |  |  |
| … |  |  |  |  |
| 16 | 1111 | 198.133.219.239/28 | 198.133.219.[240 - 253] | 198.133.219.254 |

| Подсеть | Название устройства | Интерфейс | IP | Маска | Шлюз |
| --- | --- | --- | --- | --- | --- |
| LAN_1 | R1 | Fa0/0 | 198.20.95.1 | 255.255.255.224 | - |
|  | Server1 | Fa0 | 198.20.95.2 |  | 198.20.95.1 |
|  | Server2 | Fa0 | 198.20.95.3 |  |  |
|  | Server3 | Fa0 | 198.20.95.4 |  |  |
| LAN_2 | R1 | Fa1/0 | 198.20.95.65 | 255.255.255.224 | - |
|  | PC2 | Fa0 | 198.20.95.66 |  | 198.20.95.65 |
|  | PC3 | Fa0 | 198.20.95.67 |  |  |
|  | Laptop3 | Fa0 | 198.20.95.68 |  |  |
|  | Laptop2 | Fa0 | 198.20.95.69 |  |  |
|  | Laptop1 | Fa0 | 198.20.95.70 |  |  |
| LAN_3 | R1 | Fa2/0 | 198.20.95.129 | 255.255.255.224 | - |
|  | PC1 | Fa0 | 198.20.95.130 |  | 198.20.95.129 |
|  | PC4 | Fa0 | 198.20.95.131 |  |  |
|  | PC0 | Fa0 | 198.20.95.132 |  |  |
|  | Laptop4 | Fa0 | 198.20.95.133 |  |  |

R1#show startup-config

Using 759 bytes

!

version 12.2

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname R1

!

enable secret 5 $1$mERr$.GIXK7nMh5wDtc1mBeWHw/

!

interface FastEthernet0/0

description To servers

ip address 198.20.95.1 255.255.255.224

duplex auto

speed auto

!

interface FastEthernet1/0

description To users

ip address 198.20.95.65 255.255.255.224

duplex auto

speed auto

!

interface FastEthernet2/0

description To admins

ip address 198.20.95.129 255.255.255.224

duplex auto

speed auto

!

ip classless

!

banner motd ^CHi there!^C

!

line con 0

password 0000

login

!

line aux 0

!

line vty 0 4

password 0000

login

line vty 5 15

password 0000

login

PC4

PC>ping 198.20.95.2

Pinging 198.20.95.2 with 32 bytes of data:

Reply from 198.20.95.2: bytes=32 time=1ms TTL=127

Reply from 198.20.95.2: bytes=32 time=0ms TTL=127

Reply from 198.20.95.2: bytes=32 time=0ms TTL=127

Reply from 198.20.95.2: bytes=32 time=0ms TTL=127

Ping statistics for 198.20.95.2:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss),

Approximate round trip times in milli-seconds:

Minimum = 0ms, Maximum = 1ms, Average = 0ms

Laptop2

PC>ping 198.20.95.3

Pinging 198.20.95.3 with 32 bytes of data:

Request timed out.

Reply from 198.20.95.3: bytes=32 time=0ms TTL=127

Reply from 198.20.95.3: bytes=32 time=0ms TTL=127

Reply from 198.20.95.3: bytes=32 time=1ms TTL=127

Ping statistics for 198.20.95.3:

Packets: Sent = 4, Received = 3, Lost = 1 (25% loss),

Approximate round trip times in milli-seconds:

Minimum = 0ms, Maximum = 1ms, Average = 0ms

Laptop4

PC>ping 198.20.95.70

Pinging 198.20.95.70 with 32 bytes of data:

Request timed out.

Reply from 198.20.95.70: bytes=32 time=0ms TTL=127

Reply from 198.20.95.70: bytes=32 time=1ms TTL=127

Reply from 198.20.95.70: bytes=32 time=0ms TTL=127

Ping statistics for 198.20.95.70:

Packets: Sent = 4, Received = 3, Lost = 1 (25% loss),

Approximate round trip times in milli-seconds:

Minimum = 0ms, Maximum = 1ms, Average = 0ms

Server3

SERVER>ping 198.20.95.68

Pinging 198.20.95.68 with 32 bytes of data:

Reply from 198.20.95.68: bytes=32 time=0ms TTL=127

Reply from 198.20.95.68: bytes=32 time=0ms TTL=127

Reply from 198.20.95.68: bytes=32 time=12ms TTL=127

Reply from 198.20.95.68: bytes=32 time=0ms TTL=127

Ping statistics for 198.20.95.68:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss),

Approximate round trip times in milli-seconds:

Minimum = 0ms, Maximum = 12ms, Average = 3ms

PC3

PC>ping 198.20.95.69

Pinging 198.20.95.69 with 32 bytes of data:

Reply from 198.20.95.69: bytes=32 time=1ms TTL=128

Reply from 198.20.95.69: bytes=32 time=0ms TTL=128

Reply from 198.20.95.69: bytes=32 time=0ms TTL=128

Reply from 198.20.95.69: bytes=32 time=0ms TTL=128

Ping statistics for 198.20.95.69:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss),

Approximate round trip times in milli-seconds:

Minimum = 0ms, Maximum = 1ms, Average = 0ms

R1#show arp

Protocol  Address          Age (min)  Hardware Addr   Type   Interface

Internet  198.20.95.1             -   00D0.FF75.3C10  ARPA   FastEthernet0/0

Internet  198.20.95.2             63  0010.1199.1DCD  ARPA   FastEthernet0/0

Internet  198.20.95.3             45  00E0.A318.1161  ARPA   FastEthernet0/0

Internet  198.20.95.4             43  0006.2A6A.37A7  ARPA   FastEthernet0/0

Internet  198.20.95.65            -   0006.2AB4.9D1E  ARPA   FastEthernet1/0

Internet  198.20.95.66            58  0002.4A7E.A375  ARPA   FastEthernet1/0

Internet  198.20.95.68            58  0003.E4A9.47DD  ARPA   FastEthernet1/0

Internet  198.20.95.69            55  00D0.9717.BB0B  ARPA   FastEthernet1/0

Internet  198.20.95.70            44  00D0.BA82.47B1  ARPA   FastEthernet1/0

Internet  198.20.95.129           -   0090.2188.4908  ARPA   FastEthernet2/0

Internet  198.20.95.130           55  0001.C99E.E2C5  ARPA   FastEthernet2/0

Internet  198.20.95.131           48  00D0.D398.E676  ARPA   FastEthernet2/0

Internet  198.20.95.133           56  0060.3EC5.1327  ARPA   FastEthernet2/0

Server1

SERVER>arp -a

Internet Address      Physical Address      Type

198.20.95.1           00d0.ff75.3c10        dynamic

198.20.95.3           00e0.a318.1161        dynamic

Server3

SERVER>arp -a

Internet Address      Physical Address      Type

198.20.95.1           00d0.ff75.3c10        dynamic

PC1

PC>arp -a

Internet Address      Physical Address      Type

198.20.95.133         0060.3ec5.1327        dynamic

Laptop2

PC>arp -a

Internet Address      Physical Address      Type

198.20.95.65          0006.2ab4.9d1e        dynamic

198.20.95.67          000a.f3c7.8431        dynamic